Table 1. Detection prevalence, exceedance prevalence, and detected concentrations of regulated and unregulated analytes in the EPA UCMR 5 analytic dataset.

(n = 14,071 ZIP codes; 40,311 compound-level records; 4,885 unique water systems)

| Compound | Applicable Threshold (µg/L) | ZIP Codes Detected, n (%) | Exceedance, n (% of detected) | Mean Detected Concentration (µg/L) | Maximum Detected Concentration (µg/L) |
| --- | --- | --- | --- | --- | --- |
| Regulated PFAS |  |  |  |  |  |
| PFOA | 0.004 (MCL) | 3,936 (28.0%) | 3,936 (100.0%)† | 0.0095 | 0.235 |
| PFOS | 0.004 (MCL) | 4,083 (29.0%) | 4,083 (100.0%)† | 0.0120 | 0.490 |
| PFHxS | 0.010 (MCL) | 2,999 (21.3%) | 635 (21.2%) | 0.0088 | 0.2502 |
| PFBS | 2.0 (HI ref.)‡ | 5,009 (35.6%) | —‡ | 0.0087 | 0.290 |
| PFNA | 0.010 (MCL) | 558 (4.0%) | 194 (34.8%) | 0.0096 | 0.0998 |
| HFPO-DA (GenX) | 0.010 (MCL) | 203 (1.4%) | 71 (35.0%) | 0.0209 | 0.140 |
| Unregulated Analytes |  |  |  |  |  |
| Lithium | 7.36 (ref.)§ | 8,076 (57.4%) | 8,076 (100.0%)§ | 47.6 | 960 |
| PFPeA | — | 6,396 (45.5%) | — | 0.0111 | 0.358 |
| PFBA | — | 6,065 (43.1%) | — | 0.0116 | 1.70 |
| PFHpA | — | 2,056 (14.6%) | — | 0.0067 | 0.0727 |
| 6:2 FTS | — | 896 (6.4%) | — | 0.0437 | 0.672 |
| PFDA | — | 34 (0.2%) | — | 0.0045 | 0.0124 |

All statistics were derived from the processed EPA UCMR 5 analytic dataset described in Methods. Detection percentages reflect the proportion of all 14,071 ZIP codes in the dataset. Exceedance percentages reflect the proportion of ZIP codes in which that compound was detected. The highest observed concentration per compound per ZIP code was retained; values represent the upper bound of observed drinking-water concentrations.

Abbreviations: MCL = Maximum Contaminant Level; HI = Hazard Index; ref. = reference level.
† For PFOA and PFOS, the UCMR 5 minimum reporting level equals the MCL (0.004 µg/L); every recorded detection therefore meets the definition of exceedance. Detection prevalence is reported because an independent exceedance rate cannot be calculated from the UCMR 5 reporting structure.
‡ PFBS is regulated via a combined Hazard Index requiring co-located multi-compound sample measurements; a full Hazard Index calculation was not feasible from this dataset. Exceedance is not reported for PFBS. The threshold shown (2.0 µg/L) is the individual PFBS reference concentration used in the Hazard Index calculation.
§ Lithium has no federal Maximum Contaminant Level. The threshold shown (7.36 µg/L) is the drinking-water concentration evaluated in Liew et al. (2023); it is not a regulatory standard. Figures in the exceedance column reflect the proportion of detected ZIP codes at or above this evidence-based reference level.